| EdgeCore-1 Edge AI Inference Accelerator IP Simulation-Verified RTL ¦ 28nm ¦ INT8 ¦ 500 MHz | INFINAR SCIENTIA IP Licensing infinar-scientia.com |
| --- | --- |

| Process | Die Area | Peak Compute | Power | DRAM I/F | Controller |
| --- | --- | --- | --- | --- | --- |
| 28nm TSMC/SMIC | 5.00 mm² | 32.8 TOPS INT8 | <185 mW | LPDDR5 | RISC-V RV32IMC |

| Architecture Highlights Block; Specification. MAC Array; 256×256 INT8 systolic. Tile Size; 8×8 (64 MACs/cycle). Weight Cache; 512 KB dual-bank SRAM. Sparsity Engine; Bitmap-mask, 0.27 mm². Sparsity SRAM; 32 KB, 4096 tile masks. Clock Gating; MAC + PHY, per-domain. PHY Power Mode; JEDEC deep power-down. Idle Power; 2–5 mW (PHY off). Frequency; 500 MHz (target). Verification Status Check; Result; Detail. C++ Golden Model; ✓ PASS; 10M+ tests. RTL Unit Testbench; ✓ PASS; 15/15 tests. Full System Testbench; ✓ PASS; 10/10 tests. Sparsity Verification; ✓ PASS; 5 vectors. Yosys Synthesis; ✓ PASS; 4,710 gates. Power Budget; ✓ PASS; <185 mW. FPGA Prototype; Planned; Q3 2026. |  | Why EdgeCore-1? The $4–8 gap in the market Coral Edge TPU: $5+ (overkill for most IoT). MCU: $0.50–2 (no AI HW). EdgeCore-1 targets $1.50 silicon cost at 500K units — filling the gap.. Sparsity-aware: up to 80% tile suppression Bitmap-mask sparsity engine skips zero-weight tiles before dispatch. Pruned MobileNetV2 achieves 3–4× effective throughput over dense execution.. ARM/NVIDIA style licensing — not product sales You own your silicon. Infinar provides verified RTL + compiler. You integrate, tape out, and sell chips under your brand.. Software Stack - ONNX frontend — zero custom model code - Apache TVM backend — edgecore1_target.py - Runtime driver — Python + RISC-V C firmware - PyTorch → running inference in <1 hour - MobileNetV2, MobileNetV3, EfficientNet-Lite supported Target Applications - ADAS: pedestrian detection, sign recognition - Industrial: predictive maintenance, anomaly detection - Consumer IoT: keyword spotting, image classification - Smart cameras: always-on vision at <5 mW idle Licensing Terms Term; Range. NRE Fee; $10K – $200K. Per-Unit Royalty; $0.02 – $0.15/chip. Flat License; $25K – $75K. Support (post-NRE); $250/hour. Warranty; 90-day bug-fix. |
| --- | --- | --- |

Ready to evaluate? NDA required before IP delivery. Contact: licensing@infinar-scientia.com

This document contains confidential and proprietary information of Infinar Scientia Inc. | April 2026